COMPANY IDENTITY:  INTERNATIONAL DYE & CHEMICAL                     SDS DATE: 03/27/2015

PRODUCT IDENTITY:  FABRIC SHOE DYE                                  REPLACES: 11/11/2011

SAFETY DATA SHEET

This Safety Data Sheet conforms to ANSI Z400.5, and to the

format requirements of the Global Harmonizing System.

THIS SDS COMPLIES WITH 29 CFR 1910.1200 (HAZARD COMMUNICATION STANDARD)

IMPORTANT: Read this SDS before handling & disposing of this product.

Pass this information on to employees, customers, & users of this product.

SECTION 1. IDENTIFICATION OF THE SUBSTANCE OR MIXTURE AND OF THE SUPPLIER

PRODUCT IDENTITY: FABRIC SHOE DYE

SYNONYMS:         None

PRODUCT USES:     Shoe Dye

COMPANY IDENTITY: INTERNATIONAL DYE & CHEMICAL

COMPANY ADDRESS:  901 BUSHKILL DRIVE

COMPANY CITY:     EASTON, PA 18042

COMPANY PHONE:    1-610-253-9400

EMERGENCY PHONES: CHEMTREC: 1-800-424-9300 (USA)

CANUTEC:  1-613-996-6666 (CANADA)

SECTION 2. HAZARDS IDENTIFICATION

WARNING!

2.1 HAZARD STATEMENTS: (CAT = Hazard Category)

(H200s) PHYSICAL: Flammable Liquids(CAT:3)

H226 COMBUSTIBLE LIQUID(N.America);

FLAMMABLE LIQUID & VAPOR(Elsewhere).

(H300s) HEALTH: Acute Toxicity, Oral(CAT:4)

H302 HARMFUL IF SWALLOWED.

(H300s) HEALTH: Skin Corrosion/Irritation(CAT:2)

H315 CAUSES SKIN IRRITATION.

(H300s) HEALTH: Serious Eye Damage/Eye Irritation(CAT:2)

H320 CAUSES EYE IRRITATION.

(H300s) HEALTH: Acute Toxicity, Inhalation(CAT:4)

H332 HARMFUL IF INHALED.

(H300s) HEALTH: Target Organ Toxicity, Single Exposure(CAT:3)

H335 MAY CAUSE RESPIRATORY IRRITATION.

H336 MAY CAUSE DROWSINESS OR DIZZINESS.

(H300s) HEALTH: Target Organ Toxicity, Single Exposure(CAT:2)

H371 MAY CAUSE DAMAGE TO ORGANS. (See Section 11 for Target Organ Information)

2.2 PRECAUTIONARY STATEMENTS:

EXPOSURE PREVENTION:

P100s = General, P200s = Prevention, P300s = Response, P400s = Storage, P500s = Disposal

P210         Keep away from heat/sparks/open flames/hot surfaces -- No Smoking.

P240         Ground/bond container and receiving equipment.

P241         Use explosion-proof electrical/ventilating/lighting equipment.

P242         Use only non-sparking tools.

P243         Take precautionary measures against static discharge.

P264         Wash with soap & water thoroughly after handling.

P270         Do not eat, drink or smoke when using this product.

P271         Use only outdoors or in a well-ventilated area.

P280         Wear protective gloves/protective clothing/eye protection/face protection.

P301+312     IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell.

P302+352     IF ON SKIN: Wash with soap & water.

P304+340     IF INHALED: Remove victim to fresh air & keep at rest in a position

comfortable for breathing.

P305+351+338 IF IN EYES: Rinse cautiously with water for several minutes.

Remove contact lenses if present & easy to do - Continue rinsing.

P309+311     If exposed or you feel unwell: Call a POISON CENTER or doctor/physician.

P330         Rinse mouth.

P332+313     If skin irritation occurs: Get medical advice/attention.

P337+313     If eye irritation persists, get medical advice/attention.

P361         Remove/Take off immediately all contaminated clothing.

P363         Wash contaminated clothing before reuse.

P405         Store locked up.

P500         Dispose of contents/container following local/regional/federal regulations.

SEE SECTIONS 8, 11 & 12 FOR TOXICOLOGICAL INFORMATION.

COMPANY IDENTITY:  INTERNATIONAL DYE & CHEMICAL                     SDS DATE: 03/27/2015

PRODUCT IDENTITY:  FABRIC SHOE DYE                                  REPLACES: 11/11/2011

SECTION 3. COMPOSITION/INFORMATION ON INGREDIENTS

MATERIAL                                CAS#          EINECS#        WT %

Water                             7732-18-5        231-791-2      75-85

Isopropanol                         67-63-0        200-661-7       5-15

2-Butoxyethanol                    111-76-2        203-905-0       0-10

The specific chemical component identities and/or the exact component percentages of

this material may be withheld as trade secrets. This information is made available

to health professionals, employees, and designated representatives in accordance with

the applicable provisions of 29 CFR 1910.1200 (I)(1).

TRACE COMPONENTS: Trace ingredients (if any) are present in < 1% concentration,

(< 0.1% for potential carcinogens, reproductive toxins, respiratory tract mutagens,

and sensitizers). None of the trace ingredients contribute significant additional

hazards at the concentrations that may be present in this product. All pertinent hazard

information has been provided in this document, per the requirements of the Federal

Occupational Safety and Health Administration Standard (29 CFR 1910.1200), U.S. State

equivalents, and Canadian Hazardous Materials Identification System Standard (CPR 4).

SECTION 4. FIRST AID MEASURES

4.1 MOST IMPORTANT SYMPTOMS/EFFECTS, ACUTE & CHRONIC:

See Section 11 for symptoms/effects, acute & chronic.

4.2 GENERAL ADVICE:

First Aid responders should pay attention to self-protection and use the recommended

protective clothing (chemical resistant gloves, splash protection). If potential for

exposure exists, refer to Section 8 for specific personal protective equipment.

4.3 EYE CONTACT:

If this product enters the eyes, check for and remove any contact lenses. Open eyes while

under gently running water. Use sufficient force to open eyelids. "Roll" eyes to expose

more surface. Minimum flushing is for 15 minutes. Seek immediate medical attention.

4.4 SKIN CONTACT:

If the product contaminates the skin, immediately begin decontamination with running

water. Minimum flushing is for 15 minutes. Remove contaminated clothing, taking care not

to contaminate eyes. If skin becomes irritated and irritation persists, medical attention

may be necessary. Wash contaminated clothing before reuse, discard contaminated shoes.

4.5 INHALATION:

After high vapor exposure, remove to fresh air. If breathing is difficult, give

oxygen.  If breathing has stopped, trained personnel should immediately begin artificial

respiration. If the heart has stopped, trained personnel should immediately begin

cardiopulmonary resuscitation (CPR). Seek immediate medical attention.

4.6 SWALLOWING:

If swallowed, CALL PHYSICIAN OR POISON CONTROL CENTER FOR MOST CURRENT INFORMATION. If

professional advice is not available, give two glasses of water to drink. DO NOT INDUCE

VOMITING. Never induce vomiting or give liquids to someone who is unconscious, having

convulsions, or unable to swallow. Seek immediate medical attention.

4.7 RESCUERS: Victims of chemical exposure must be taken for medical attention. Rescuers

should be taken for medical attention, if necessary. Take a copy of label and SDS to

physician or health professional with victim.

4.8 NOTES TO PHYSICIAN:

There is no specific antidote. Treatment of overexposure should be directed at the control

of symptoms and the clinical condition of the patient. Any material aspirated during

vomiting may cause lung injury. Therefore, emesis should not be induced mechanically or

pharmacologically. If it is considered necessary to evacuate the stomach contents, this

should be done by means least likely to cause aspiration (such as: Gastric lavage after

endotracheal intubation).

SECTION 5. FIRE FIGHTING MEASURES

5.1 FIRE & EXPLOSION PREVENTIVE MEASURES:

NO open flames, NO sparks, & NO smoking. Above flash point, use

a closed system, ventilation, explosion-proof electrical equipment, lighting.

COMPANY IDENTITY:  INTERNATIONAL DYE & CHEMICAL                     SDS DATE: 03/27/2015

PRODUCT IDENTITY:  FABRIC SHOE DYE                                  REPLACES: 11/11/2011

SECTION 5. FIRE FIGHTING MEASURES (CONTINUED)

5.2 SUITABLE (& UNSUITABLE) EXTINGUISHING MEDIA:

Use dry powder, alcohol-resistant foam, water spray, water in large amounts,

carbon dioxide.

5.3 SPECIAL PROTECTIVE EQUIPMENT & PRECAUTIONS FOR FIRE FIGHTERS:

Water spray may be ineffective on fire but can protect fire-fighters

& cool closed containers. Use fog nozzles if water is used.

Do not enter confined fire-space without full bunker gear.

(Helmet with face shield, bunker coats, gloves & rubber boots).

5.4 SPECIFIC HAZARDS OF CHEMICAL & HAZARDOUS COMBUSTION PRODUCTS:

COMBUSTIBLE!

Isolate from oxidizers, heat, & open flame.

Closed containers may explode if exposed to extreme heat.

Applying to hot surfaces requires special precautions.

Empty container very hazardous! Continue all label precautions!

SECTION 6. ACCIDENTAL RELEASE MEASURES

6.1 SPILL AND LEAK RESPONSE AND ENVIRONMENTAL PRECAUTIONS:

Uncontrolled releases should be responded to by trained personnel using pre-planned

procedures. No action shall be taken involving personal risk without suitable training.

Keep unnecessary and unprotected personnel from entering spill area. Do not touch or

walk through material. Avoid breathing vapor or mist. Provide adequate ventilation.

Proper protective equipment should be used. In case of a spill, clear the affected area,

protect people, and respond with trained personnel. ELIMINATE all ignition sources

(no smoking, flares, sparks, or flames in immediate area).

6.2 PERSONAL PRECAUTIONS, PROTECTIVE EQUIPMENT, EMERGENCY PROCEDURES:

The proper personal protective equipment for incidental releases (such as: 1 Liter of the

product released in a well-ventilated area), use impermeable gloves, they should be Level

B: triple-gloves (rubber gloves and nitrile gloves over latex gloves), chemical resistant

suit and boots, hard-hat, and Self-Contained Breathing Apparatus specific for the material

handled, goggles, face shield, and appropriate body protection. In the event of a large

release, use impermeable gloves, specific for the material handled, chemically resistant

suit and boots, and hard hat, and Self-Contained Breathing Apparatus or respirator.

Personal protective equipment are required wherever engineering controls are not adequate

or conditions for potential exposure exist. Select NIOSH/MSHA approved based on actual

or potential airborne concentrations in accordance with latest OSHA and/or ANSI

recommendations.

6.3 ENVIRONMENTAL PRECAUTIONS:

Stop spill at source. Construct temporary dikes of dirt, sand, or any appropriate readily

available material to prevent spreading of the material. Close or cap valves and/or

block or plug hole in leaking container and transfer to another container. Keep from

entering storm sewers and ditches which lead to waterways, and if necessary, call the

local fire or police department for immediate emergency assistance.

6.4 METHODS AND MATERIAL FOR CONTAINMENT & CLEAN-UP:

Absorb spilled liquid with polypads or other suitable absorbent materials. If necessary,

neutralize using suitable buffering material, (acid with soda ash or base with phosphoric

acid), and test area with litmus paper to confirm neutralization. Clean up with

non-combustible absorbent (such as: sand, soil, and so on). Shovel up and place all spill

residue in suitable containers. dispose of at an appropriate waste disposal facility

according to current applicable laws and regulations and product characteristics at time

of disposal (see Section 13 - Disposal Considerations).

6.5 NOTIFICATION PROCEDURES:

In the event of a spill or accidental release, notify relevant authorities in accordance

with all applicable regulations. US regulations require reporting release of this material

to the environment which exceed the applicable reportable quantity or oil spills which could

reach any waterway including intermittent dry creeks. The National Response Center can be

reached at (800)424-8802.

COMPANY IDENTITY:  INTERNATIONAL DYE & CHEMICAL                     SDS DATE: 03/27/2015

PRODUCT IDENTITY:  FABRIC SHOE DYE                                  REPLACES: 11/11/2011

SECTION 7. HANDLING AND STORAGE

7.1 PRECAUTIONS FOR SAFE HANDLING:

Isolate from oxidizers, heat, & open flame. Use only with adequate ventilation. Avoid

prolonged or repeated breathing of vapor or spray mist. Do not get in eyes, on skin or

clothing. Consult Safety Equipment Supplier. Wear goggles, face shield, gloves, apron

& footwear impervious to material. Wash clothing before reuse.

Avoid free fall of liquid. Ground containers when transferring. Do not flame cut,

braze, or weld. Empty container very hazardous! Continue all label precautions!

7.2 CONDITIONS FOR SAFE STORAGE, INCLUDING ANY INCOMPATIBILITIES:

Keep in fireproof surroundings. Keep separated from strong oxidants. Keep cool.

Do not store above 49 C/120 F.

Keep container tightly closed & upright when not in use to prevent leakage.

7.3 NONBULK: CONTAINERS:

Store containers in a cool, dry location, away from direct sunlight, sources of intense

heat, or where freezing is possible. Material should be stored in secondary containers or

in a diked area, as appropriate. Store containers away from incompatible chemicals (see

Section 10, Stability and Reactivity). Post warning and "NO SMOKING" signs in storage and

use areas, as appropriate. Empty containers should be handled with care. Never store food,

feed, or drinking water in containers which held this product.

7.4 BULK CONTAINERS:

All tanks and pipelines which contain this material must be labeled. Perform routine

maintenance on tanks or pipelines which contain this product. Report all leaks immediately

to the proper personnel.

7.5 TANK CAR SHIPMENTS:

Tank cars carrying this product should be loaded and unloaded in strict accordance with

tank-car manufacturer's recommendation and all established on-site safety procedures.

Appropriate personal protective equipment must be used (see Section 8, Engineering

Controls and Personal Protective Equipment.). All loading and unloading equipment must be

inspected, prior to each use. Loading and unloading operations must be attended, at all

times. Tank cars must be level, brakes must be set or wheels must be locked or blocked

prior to loading or unloading. Tank car (for loading) or storage tanks (for unloading)

must be verified to be correct for receiving this product and be properly prepared,

prior to starting the transfer operations. Hoses must be verified to be in the correct

positions, before starting transfer operations. A sample (if required) must be taken

and verified (if required) prior to starting transfer operations. All lines must be

blown-down and purged before disconnecting them from the tank car or vessel.

7.6 PROTECTIVE PRACTICES DURING MAINTENANCE OF CONTAMINATED EQUIPMENT:

Follow practices indicated in Section 6 (Accidental Release Measures). Make certain

application equipment is locked and tagged-out safely. Always use this product in areas

where adequate ventilation is provided. Collect all rinsates and dispose of according

to applicable Federal, State, Provincial, or local procedures.

7.7 EMPTY CONTAINER WARNING:

Empty containers may contain residue and can be dangerous. Do not attempt to refill or

clean containers without proper instructions. Empty drums should be completely drained

and safely stored until appropriately reconditioned or disposed. Empty containers should

be taken for recycling, recovery, or disposal through suitably qualified or licensed

contractor and in accordance with governmental regulations. DO NOT PRESSURIZE, CUT, WELD,

BRAZE, SOLDER, DRILL, GRIND, OR EXPOSE SUCH CONTAINERS TO HEAT, FLAME, SPARKS, STATIC

ELECTRICITY, OR OTHER SOURCES OF IGNITION. THEY MAY BURST AND CAUSE INJURY OR DEATH.

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

8.1 EXPOSURE LIMITS:

MATERIAL                             CAS#     EINECS#  TWA (OSHA)     TLV (ACGIH)

Water                          7732-18-5   231-791-2  None Known     None Known

Isopropanol                      67-63-0   200-661-7   400 ppm        200 ppm A4

2-Butoxyethanol                 111-76-2   203-905-0    50 ppm S       20 ppm S

MATERIAL                        CAS#     EINECS#   CEILING  STEL(OSHA/ACGIH)  HAP

Isopropanol                   67-63-0  200-661-7  None Known   400 ppm        No

This product contains no EPA Hazardous Air Pollutants (HAP) in amounts > 0.1%.

COMPANY IDENTITY:  INTERNATIONAL DYE & CHEMICAL                     SDS DATE: 03/27/2015

PRODUCT IDENTITY:  FABRIC SHOE DYE                                  REPLACES: 11/11/2011

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION (CONTINUED)

8.2 APPROPRIATE ENGINEERING CONTROLS:

RESPIRATORY EXPOSURE CONTROLS

Airborne concentrations should be kept to lowest levels possible. If vapor, dust or mist

is generated and the occupational exposure limit of the product, or any component of the

product, is exceeded, use appropriate NIOSH or MSHA approved air purifying or air-supplied

respirator authorized in 29 CFR 1910.134, European Standard EN 149, or applicable State

regulations, after determining the airborne concentration of the contaminant. Air supplied

respirators should always be worn when airborne concentration of the contaminant or

oxygen content is unknown. Maintain airborne contaminant concentrations below exposure

limits. If adequate ventilation is not available or there is potential for airborne

exposure above the exposure limits, a respirator may be worn up to the respirator exposure

limitations, check with respirator equipment manufacturer's recommendations/limitations.

For particulates, a particulate respirator (NIOSH Type N95 or better filters) may be worn.

If oil particles (such as: lubricants, cutting fluids, glycerine, and so on) are present,

use a NIOSH Type R or P filter. For a higher level of protection, use positive pressure

supplied air respiration protection or Self-Contained Breathing Apparatus or if oxygen

levels are below 19.5% or are unknown.

EMERGENCY OR PLANNED ENTRY INTO UNKNOWN CONCENTRATIONS OR IDLH CONDITIONS

Positive pressure, full-face piece Self-Contained Breathing Apparatus; or positive

pressure, full-face piece Self-Contained Breathing Apparatus with an auxilliary positive

pressure Self-Contained Breathing Apparatus.

VENTILATION

LOCAL EXHAUST:   Necessary          MECHANICAL (GENERAL):  Necessary

SPECIAL:         None               OTHER:                 None

Please refer to ACGIH document, "Industrial Ventilation, A Manual of

Recommended Practices", most recent edition, for details.

8.3 INDIVIDUAL PROTECTION MEASURES, SUCH AS PERSONAL PROTECTIVE EQUIPMENT:

EYE PROTECTION:

Safety eyewear complying with an approved standard should be used when a risk

assessment indicates this is necessary to avoid exposure to liquid splashes, mists or

dusts. If contact is possible, chemical splash goggles should be worn, when a higher

degree of protection is  necessary, use splash goggles or safety glasses. Face-shields

are recommended when the operation can generate splashes, sprays or mists.

HAND PROTECTION:

Use gloves chemically resistant to this material. Glove must be inspected prior to use.

Preferred examples: Butyl rubber, Chlorinated Polyethylene, Polyethylene, Ethyl vinyl alcohol

laminate ("EVAL"), Polyvinyl alcohol ("PVA"). Examples of acceptable glove barrier materials

include: Natural rubber ("latex"), Neoprene, Nitrile/butadiene rubber ("nitrile") or ("NBR"),

Polyvinyl chloride ("PVC") or "vinyl"), Viton. Chemical-resistant, impervious gloves complying

with an approved standard should be worn at all times when handling chemical products if a

risk assessment indicates this is necessary. Considering the parameters specified by the glove

manufacturer, check during use that the gloves are still retaining their protective properties.

It should be noted that the time to breakthrough for any glove material may be different for

different glove manufacturers. In the case of mixtures, consisting of several substances, the

protection time of the gloves cannot be accurately estimated.

Use proper glove removal technique (without touching glove's outer surface) to avoid skin

contact with this product. Dispose of contaminated gloves after use in accordance with

applicable laws and good practices. Wash and dry hands.

BODY PROTECTION:

Use body protection appropriate for task. Cover-all, rubber aprons, or chemical protective

clothing made from impervious materials are generally acceptable, depending on the task.

WORK & HYGIENIC PRACTICES:

Wash hands, forearms and face thoroughly after handling chemical products, before eating,

smoking and using toilet facilities and at the end of the working period. Provide readily

accessible eye wash stations & safety showers. Remove clothing that becomes contaminated.

Destroy contaminated leather articles. Launder or discard contaminated clothing.

COMPANY IDENTITY:  INTERNATIONAL DYE & CHEMICAL                     SDS DATE: 03/27/2015

PRODUCT IDENTITY:  FABRIC SHOE DYE                                  REPLACES: 11/11/2011

SECTION 9. PHYSICAL & CHEMICAL PROPERTIES

APPEARANCE:                                       Liquid, Different Colors

ODOR:                                             Alcohol

ODOR THRESHOLD:                                   Not Available

pH (Neutrality):                                  Not Applicable

MELTING POINT/FREEZING POINT:                     Not Available

BOILING RANGE (IBP,50%,Dry Point):                80 103 172 C / 177 218 342 F

FLASH POINT (TEST METHOD):                        > 38 C / > 101 F (PMCC)

EVAPORATION RATE (n-BUTYL ACETATE=1):             0.241

FLAMMABILITY CLASSIFICATION:                      Class II

LOWER FLAMMABLE LIMIT IN AIR (% by vol):          1.1 (Lowest Component)

UPPER FLAMMABLE LIMIT IN AIR (% by vol):          Not Available

VAPOR PRESSURE (mm of Hg)@20 C                    17.9

VAPOR DENSITY (air=1):                            0.776

GRAVITY @ 68/68 F / 20/20 C:

DENSITY:                                       0.959

SPECIFIC GRAVITY (Water=1):                    0.960

POUNDS/GALLON:                                 7.997

WATER SOLUBILITY:                                 Complete

PARTITION COEFFICIENT (n-Octane/Water):           Not Available

AUTO IGNITION TEMPERATURE:                        398 C /  750 F

DECOMPOSITION TEMPERATURE:                        Not Available

TOTAL VOC'S (TVOC)*:                              22.0 Vol% / 183.5 g/L / 1.5 Lbs/Gal

NONEXEMPT VOC'S (CVOC)*:                          22.0 Vol% / 183.5 g/L / 1.5 Lbs/Gal

HAZARDOUS AIR POLLUTANTS (HAPS):                  0.0  Wt% /0.0 g/L / 0.000 Lbs/Gal

NONEXEMPT VOC PARTIAL PRESSURE (mm of Hg @ 20 C)  1.2

VISCOSITY @ 20 C (ASTM D445):                     Not Available

* Using CARB (California Air Resources Board Rules).

SECTION 10. STABILITY & REACTIVITY

10.1 REACTIVITY & CHEMICAL STABILITY:

Stable under normal conditions, no hazardous reactions when kept from incompatibles.

10.2 POSSIBILITY OF HAZARDOUS REACTIONS & CONDITIONS TO AVOID:

Isolate from oxidizers, heat, & open flame.

10.3 INCOMPATIBLE MATERIALS:

Reacts with strong oxidants, causing fire & explosion hazard. Attacks

10.4 HAZARDOUS DECOMPOSITION PRODUCTS:

Carbon Monoxide, Carbon Dioxide from burning.

10.5 HAZARDOUS POLYMERIZATION:

Will not occur.

SECTION 11. TOXICOLOGICAL INFORMATION

11.1 ACUTE HAZARDS

11.1.1 SKIN CONTACT:

Primary irritation to skin, defatting, dermatitis.

Absorption thru skin increases exposure.

Wash thoroughly after handling.

11.1.2 EYE CONTACT:

Primary irritation to eyes, redness, tearing, blurred vision.

Liquid can cause eye irritation.

11.1.3 INHALATION:

Anesthetic. Irritates respiratory tract. Acute overexposure

can cause serious nervous system depression. Vapor harmful.

Breathing vapor can cause irritation.

Acute overexposure can cause harm to affected organs by routes of entry.

11.1.4 SWALLOWING:

Swallowing can cause abdominal irritation, nausea, vomiting & diarrhea.

COMPANY IDENTITY:  INTERNATIONAL DYE & CHEMICAL                     SDS DATE: 03/27/2015

PRODUCT IDENTITY:  FABRIC SHOE DYE                                  REPLACES: 11/11/2011

SECTION 11. TOXICOLOGICAL INFORMATION (CONTINUED)

11.2 SUBCHRONIC HAZARDS/CONDITIONS AGGRAVATED

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE:

Pre-existing disorders of any target organs mentioned in this Document can be

aggravated by over-exposure by routes of entry to components of this product.

Persons with these disorders should avoid use of this product.

11.3 CHRONIC HAZARDS

11.3.1 CANCER, REPRODUCTIVE & OTHER CHRONIC HAZARDS:

This product has no carcinogens listed by IARC, NTP, NIOSH,

OSHA or ACGIH, as of this date, greater or equal to 0.1%.

Absorption thru skin may be harmful.

11.3.2 TARGET ORGANS: May cause damage to target organs, based on animal data.

11.3.3 IRRITANCY: Irritating to contaminated tissue.

11.3.4 SENSITIZATION: No component is known as a sensitizer.

11.3.5 MUTAGENICITY: No known reports of mutagenic effects in humans.

11.3.6 EMBRYOTOXICITY: No known reports of embryotoxic effects in humans.

11.3.7 TERATOGENICITY: No known reports of teratogenic effects in humans.

11.3.8 REPRODUCTIVE TOXICITY: No known reports of reproductive effects in humans.

A MUTAGEN is a chemical which causes permanent changes to genetic material (DNA)

such that the changes will propagate across generational lines. An EMBRYOTOXIN

is a chemical which causes damage to a developing embryo (such as: within the first

8 weeks of pregnancy in humans), but the damage does not propagate across

generational lines. A TERATOGEN is a chemical which causes damage to a developing

fetus, but the damage does not propagate across generational lines. A REPRODUCTIVE

TOXIN is any substance which interferes in any way with the reproductive process.

11.4 MAMMALIAN TOXICITY INFORMATION

MATERIAL                        CAS#     EINECS#     LOWEST KNOWN LETHAL DOSE DATA

LOWEST KNOWN LD50 (ORAL)

Ethylene Glycol Butyl Ether   111-76-2  203-905-0    320.0 mg/kg(Rabbits)

LOWEST KNOWN LC50 (VAPORS)

Ethylene Glycol Butyl Ether   111-76-2  203-905-0    700 ppm (Mice)

LOWEST KNOWN LD50 (SKIN)

Ethylene Glycol Butyl Ether   111-76-2  203-905-0    440.0 mg/kg (Rabbits)

SECTION 12. ECOLOGICAL INFORMATION

12.1 ALL WORK PRACTICES MUST BE AIMED AT ELIMINATING ENVIRONMENTAL CONTAMINATION.

12.2 EFFECT OF MATERIAL ON PLANTS AND ANIMALS:

This product may be harmful or fatal to plant and animal life if released

into the environment. Refer to Section 11 (Toxicological Information) for

further data on the effects of this product's components on test animals.

12.3 EFFECT OF MATERIAL ON AQUATIC LIFE:

The most sensitive known aquatic group to any component of this product is:

Tidewater Silversides 1000 ppm or mg/L (24 hour exposure).

Keep out of sewers and natural water supplies.

12.4 MOBILITY IN SOIL

This material is a mobile liquid.

12.5 DEGRADABILITY

This product is partially biodegradable.

12.6 ACCUMULATION

This product does not accumulate or biomagnify in the environment.

COMPANY IDENTITY:  INTERNATIONAL DYE & CHEMICAL                     SDS DATE: 03/27/2015

PRODUCT IDENTITY:  FABRIC SHOE DYE                                  REPLACES: 11/11/2011

SECTION 13. DISPOSAL CONSIDERATIONS

THE GENERATION OF WASTE SHOULD BE AVOIDED OR MINIMIZED WHEREVER POSSIBLE.

Dispose of surplus and non-recyclable products via a licensed waste disposal contractor.

Waste should not be disposed of untreated to the sewer unless fully compliant with the

requirements of all authorities with jurisdiction. Waste packaging should be recycled.

Incineration or landfill should only be considered when recycling is not feasible.

This material and its container must be disposed of in a safe way. Care should be taken

when handling emptied containers that have not been cleaned or rinsed out. Empty containers

and liners may retain some product residues. Vapor from some product residues may create a

highly flammable or explosive atmosphere inside the container. DO NOT PRESSURIZE, CUT, WELD,

BRAZE, SOLDER, DRILL, GRIND, OR EXPOSE USED CONTAINERS TO HEAT, FLAME, SPARKS, STATIC

ELECTRICITY, OR OTHER SOURCES OF IGNITION. THEY MAY BURST AND CAUSE INJURY OR DEATH. Avoid

dispersal of spilled material and runoff and contact with soil, waterways, drains and sewers.

Processing, use or contamination may change the waste disposal requirements. Do not

dispose of on land, in surface waters, or in storm drains. Waste should be recycled

or disposed of in accordance with regulations. Large amounts should be collected

for reuse or consigned to licensed hazardous waste haulers for disposal.

ALL DISPOSAL MUST BE IN ACCORDANCE WITH ALL FEDERAL, STATE, PROVINCIAL, AND LOCAL

REGULATIONS. IF IN DOUBT, CONTACT PROPER AGENCIES. EPA CHARACTERISTIC: D001

SECTION 14. TRANSPORT INFORMATION

DOT SHIPPING NAME: According to: 49 CFR 173.150(d)(2) this product is classified

as a alcohol solution, combustible liquid. Because this

product is always shipped in containers of less than

66 gallons / 250 liters, other than by air transport,

it is not regulated.

EMERGENCY RESPONSE GUIDEBOOK NUMBER:  127

SECTION 15. REGULATORY INFORMATION

15.1 EPA REGULATION:

SARA SECTION 311/312 HAZARDS: Acute Health, Chronic Health, Fire

All components of this product are on the TSCA list.

SARA Title III Section 313 Supplier Notification

This product contains the indicated <*> toxic chemicals subject to the

reporting requirements of Section 313 of the Emergency Planning & Community

Right-To-Know Act of 1986 & of 40 CFR 372. This information must be

included in all MSDSs that are copied and distributed for this material.

SARA TITLE III INGREDIENTS        CAS#    EINECS#    WT%   (REG.SECTION)   RQ(LBS)

*2-Butoxyethanol              111-76-2 203-905-0   0-10  (313)              None

15.2 STATE REGULATIONS:

THIS PRODUCT MEETS REQUIREMENTS OF SOUTHERN

CALIFORNIA AQMD RULE 443.1 & SIMILAR REGULATIONS

CALIFORNIA SAFE DRINKING WATER & TOXIC ENFORCEMENT ACT (PROPOSITION 65):

This product contains no chemicals known to the State of California

to cause cancer or reproductive toxicity.

15.3 INTERNATIONAL REGULATIONS

The identified components of this product are listed on the chemical inventories

of the following countries:

Australia (AICS), Canada (DSL or  NDSL), China (IECSC), Europe (EINECS, ELINCS),

Japan (METI/CSCL, MHLW/ISHL), South Korea (KECI), New Zealand (NZIoC),

Philippines (PICCS), Switzerland (SWISS), Taiwan (NECSI), USA (TSCA).

15.4 CANADA: WORKPLACE HAZARDOUS MATERIALS INFORMATION SYSTEM (WHMIS)

B3:  Combustible Liquid.

D2B: Irritating to skin / eyes.

This product was classified using the hazard criteria of the Controlled Products

Regulations (CPR). This Document contains all information required by the CPR.

COMPANY IDENTITY:  INTERNATIONAL DYE & CHEMICAL                     SDS DATE: 03/27/2015

PRODUCT IDENTITY:  FABRIC SHOE DYE                                  REPLACES: 11/11/2011

SECTION 16. OTHER INFORMATION

16.1 HAZARD  RATINGS:

HEALTH (NFPA): 2,   HEALTH (HMIS): 2,   FLAMMABILITY: 2,   PHYSICAL HAZARD: 0

(Personal Protection Rating to be supplied by user based on use conditions.)

This information is intended solely for the use of individuals

trained in the NFPA & HMIS hazard rating systems.

16.2 EMPLOYEE TRAINING

See Section 2 (Hazards Identification). Employees should be made aware of

all hazards of this material (as stated in this SDS) before handling it.

16.3 SDS DATE: 03/27/2015

NOTICE

The supplier disclaims all expressed or implied warranties of merchantability or fitness

for a specific use, with respect to the product or the information provided herein,

except for conformation to contracted specifications. All information appearing herein

is based upon data obtained from manufacturers and/or recognized technical sources.

While the information is believed to be accurate, we make no representations as to its

accuracy or sufficiency.

Conditions of use are beyond our control, and therefore users are responsible for

verifying the data under their own operating conditions to determine whether the product

is suitable for their particular purposes and they assume all risks of their handling,

and disposal of the product. Users also assume all risks in regards to the publication

or use of, or reliance upon information contained herein.

This information relates only to the product designated herein, and does not relate to

its use in combination with any other material or process.

Unless updated, the Safety Data Sheet is valid until 03/27/2018.